## . Project Introduction

This project, titled "Solar Tracker with Automatic Cleaning System," is designed to maximize the efficiency of solar panels. While solar energy is one of the best renewable power sources available, standard solar panels lose a significant amount of their efficiency if they remain fixed in one position or if they are covered in dust and debris.

+2

To overcome these challenges, this system introduces a dual-action solution using an Arduino Nano as the central controller:

+2

- Solar Tracking: The system automatically rotates the solar panel to follow the sun's path throughout the day, ensuring the panel is always directly facing the sun to capture maximum light.

- Automatic Cleaning: It eliminates the need for manual maintenance by automatically wiping dust off the panel, guaranteeing the highest possible electricity production with zero manual effort.

## 2. Components in Detail (Viva Preparation)

For your viva evaluation, you must understand the exact technical role of each component used in the circuit setup:

- Arduino Nano: An open-source electronics platform based on easy-to-use hardware and software. It contains a programmable microcontroller chip that acts as the "brain" of the project, reading physical inputs from sensors and processing them to generate physical outputs.

+1

- Light Dependent Resistors (LDRs): Optical light sensors placed on opposite sides of the solar panel. They are required for the tracking mechanism to detect changes in light intensity.

+4

- Relay Module: An electromagnetic switch that acts as a bridge between the low-power Arduino and the high-current motor circuit, allowing power to flow safely without damaging the Arduino board.

+1

- DC Motor: A motor that provides mechanical movement to rotate the panel for tracking and to sweep a wiper or brush across the surface for cleaning.

+2

- Lithium-ion Battery: A 3.7V rechargeable power cell that serves as the main electrical energy source for the entire project.

+1

- DC-DC Boost Converter Module: A step-up voltage regulator module. Since the Arduino Nano and the relay module require exactly to operate properly, this component steps up the battery's output to a stable .

+1

- Transistor: A semiconductor device used as an electronic switch. It takes a low-power digital signal from the Arduino to trigger and activate the relay module.

+2

- Breadboard & Jumper Wires: Components used to lay out the electronic circuit and establish solderless electrical connections between all parts.

+1

- Resistors & Capacitors: Resistors are used to limit current and set up voltage dividers with the LDRs. Capacitors are used to filter electrical noise and stabilize voltages across the circuit.

+2

## 3. Working Principle Explained

The system is split into two independent functional operations managed by the Arduino Nano:

### A) Solar Tracking Mechanism (Light Response)

- The tracking system ensures that the solar panel actively tracks the light source to maintain peak power generation.

+1

- Two LDR sensors are mounted on opposite sides of the panel frame.

- When the sun moves, one LDR receives higher light intensity than the other, causing its electrical resistance to drop drastically.

+1

- This drop in resistance creates a measurable voltage difference across a voltage divider circuit.

- The Arduino constantly monitors these analog voltage changes. Upon detecting an imbalance, it activates the tracking motor to tilt or rotate the panel toward the stronger light.

+1

- The motor stops rotating once the light intensity hitting both LDRs becomes equal, keeping the panel directly aligned with the sun throughout the day.

### B) Automatic Cleaning System (Arduino Controlled)

- The Arduino Nano is programmed with a specific timer code, or it triggers based on detecting a massive drop in the solar panel's power output due to dust blockage.

+1

- When a cleaning cycle is initiated, the Arduino sends a low-power digital signal to the base of the Transistor.

- The transistor acts as a switch, instantly energizing and activating the Relay Module.

- The relay contacts close, bridging the connection to allow high current from the power management circuit to flow directly into the cleaning DC Motor.

+1

- The DC motor drives a wiper or brush assembly that sweeps across the face of the solar panel to clear away accumulated dust and debris.

- Once the pre-programmed cleaning duration finishes, the Arduino cuts off the digital signal, turning off the relay and stopping the motor safely.

## 4. Relation of Arduino to Applied Physics

In a physics lab viva, examiners frequently ask how an automation project relates to core physics principles. You can explain the relationship using these four distinct pillars:

### 1. Solid-State Physics and Semiconductors

The central microcontroller chip of the Arduino Nano is built completely out of microscopic transistors fabricated on Silicon, a semiconductor material. The fundamental way this chip processes information—by controlling the movement of electrons to represent binary data (s and s)—is a direct practical application of solid-state physics and electronic band theory.

+1

### 2. Electricity & Circuits (Ohm’s Law)

The entire electrical network relies on basic circuit dynamics. The Arduino regulates current flow and maps sensor data by manipulating voltages, currents, and resistances based strictly on Ohm's Law:

+1

Resistors are placed precisely using this law to safely distribute voltage and prevent components from burning out.

### 3. Light Physics (Photoconductive / Photoelectric Effect)

The tracking system relies heavily on light physics. When light energy (photons) hits the surface of the Light Dependent Resistor (LDR), the absorbed photon energy frees up bound electrons within the material. This physical interaction changes the material's electrical conductivity and drops its internal resistance, converting light energy into a measurable electrical property.

+2

### 4. Motion & Electromagnetism

The mechanical cleaning action relies entirely on electromagnetic principles. When the Arduino routes electricity into the coils of the DC motor via the relay, the moving electrical current generates a localized magnetic field inside the motor casing. This magnetic field interacts with permanent magnets to create a physical force (magnetic torque), converting electrical energy into the mechanical motion required to spin the wiper.

+3

## . Key Physics Terms Explained

### Electromagnetism (برقناطیسیت)

- Definition: Electromagnetism is a branch of physics that studies the relationship between electricity (electric currents) and magnetism.

- In Your Project: When electricity from the battery passes through the copper coils inside your DC motor , it generates a temporary magnetic field. This magnetic field interacts with the permanent magnets inside the motor casing, creating a rotational force (called Magnetic Torque) that spins the motor to move either the panel or the wiper.

### Photoconductive Effect (فوٹو کنڈکٹیو ایفیکٹ)

- Definition: This is an optical phenomenon where a material's electrical conductivity increases (meaning its resistance drops) when it absorbs light energy (photons).

- In Your Project: This is the exact principle behind your LDR sensors. When sunlight hits the LDR, light photons knock electrons loose inside the semiconductor material. More free electrons mean electricity can flow more easily, causing the electrical resistance to drop.

### Torque (ٹارک)

- Definition: Torque is the rotational equivalent of linear force. It is the measure of the turning force applied to an object to make it rotate around an axis.

- In Your Project: To tilt the heavy solar panel or to push the cleaning wiper across the friction-heavy surface of the glass, the DC motors must generate enough torque to overcome inertia and friction.

### Efficiency (کارکردگی)

- Definition: In energy systems, efficiency is the ratio of useful energy output to the total energy input ($\text{Efficiency} = \frac{\text{Energy Output}}{\text{Energy Input}} \times 100\%$).

- In Your Project: Solar panels have a maximum efficiency limit. Dust blocks sunlight , and misaligned angles reduce light absorption. By keeping the panel clean and perfectly aligned with the sun , your project keeps the actual output as close to that maximum theoretical efficiency as possible.

## 2. Key Electrical & Circuit Terms Explained

### Voltage ($V$)

- Definition: Also known as electric potential difference, voltage is the electrical "pressure" or driving force that pushes electric charges (electrons) through a conducting loop. It is measured in Volts (V).

- In Your Project: Your Lithium-ion battery provides a raw driving pressure of $3.7\text{V}$. Your Arduino Nano needs a higher pressure of exactly $5\text{V}$ to activate its logic gates smoothly.

### Current ($I$)

- Definition: Current is the actual rate of flow of electric charge (electrons) passing through a specific point in a circuit per second. It is measured in Amperes (A) or Amps.

- In Your Project: The Arduino pins can only handle very tiny currents (around $20\text{mA}$ to $40\text{mA}$). The DC motor requires a much larger current to spin. This is why you cannot connect the motor directly to the Arduino and must use a relay instead.

### Resistance ($R$)

- Definition: Resistance is the measure of opposition to the flow of electric current inside a material. It acts like friction against moving electrons and is measured in Ohms ($\Omega$).

- In Your Project: You use fixed resistors to safely limit current so components don't burn out. You also use variable resistors (LDRs) whose internal resistance value actively changes based on light levels.

### Ohm's Law

- Definition: A fundamental law of electricity stating that the current passing through a conductor between two points is directly proportional to the voltage across the two points, and inversely proportional to the resistance. It is written mathematically as:

$$V = IR$$

- In Your Project: Arduino calculates and tracks light changes by reading changes in voltage ($V$) that occur whenever the LDR's resistance ($R$) shifts under the sun.

## 3. Advanced Component Terms Explained

### Microcontroller

- Definition: A compact integrated circuit designed to govern a specific operation in an embedded system. It includes a processor, memory, and input/output (I/O) peripherals on a single chip.

- In Your Project: The ATmega328P chip on your Arduino Nano is the microcontroller. It takes the analog voltage signals from the LDRs , processes the mathematical difference using its code , and switches digital output pins on or off.

### Semiconductor

- Definition: A material (usually Silicon or Germanium) that has an electrical conductivity value falling between that of a conductor (like copper) and an insulator (like glass). Its conductivity can be controlled by introducing impurities or external factors like light and voltage.

- In Your Project: The transistors inside the Arduino processor and the external switching transistor are made of silicon semiconductors. They allow your circuit to act as a digital gateway, altering states between fully conducting (ON / binary 1) and insulating (OFF / binary 0).

### DC-DC Boost Converter (Step-Up Regulator)

- Definition: A switching power supply circuit that steps up (increases) a lower direct current (DC) voltage input to a higher designated DC voltage output.

- In Your Project: It uses an internal inductor (coil), capacitor, and switching diode to temporarily store energy and release it at a higher voltage level, smoothly converting your battery's $3.7\text{V}$ into the necessary $5\text{V}$.

### Electromagnetic Relay

- Definition: An electrically operated switch that uses an electromagnet to mechanically move internal metallic contacts to open or close an isolated circuit.

- In Your Project: When the Arduino sends a tiny current to the relay coil , it creates a magnetic pull that snaps a physical internal switch closed. This connects the high-current battery line directly to the DC motor, keeping your sensitive Arduino safely isolated from heavy motor electrical noise.

In Urdu ;

Yeh project "Solar Tracker with Automatic Cleaning System" ek aisi jadeed aur smart device hai jo solar panel ki efficiency ko zyada se zyada barhanay ke liye design ki gayi hai.

+2

Aam solar panels fixed hotay hain, jis ki wajah se suraj ke ghoomnay ke sath sath un par roshni kam parnay lagti hai. Is ke ilawa panels par mitti aur dhool jama honay se un ki bijli bananay ki salahiyat bohot had tak gir jati hai. Is double maslay ko hal karne ke liye is system mein do aham features shamil kiye gaye hain:

+1

- Solar Tracking: Panel suraj ke rastey ke sath khud-ba-khud rotate karta hai taake saara din zyada se zyada light capture kar sakay.

+1

- Automatic Cleaning: Panel par jami mitti ko saaf karne ke liye ek automatic wiper/brush system lagaya gaya hai jo bina kisi insani madad (zero manual effort) ke panel ko saaf rakhta hai.

+1

Is pooray system ka central controller ya "brain" Arduino Nano hai jo sensors se data ley kar motors ko chalata hai.

+1

## 2. Components in Detail (Viva Preparation)

Viva mein examinaer ke samnay detail se explain karne ke liye har component ka kaam nichey diya gaya hai:

- Arduino Nano: * Yeh ek open-source physical programmable circuit board (microcontroller) hai.

  - Project mein yeh "brain" ka kaam karta hai—yeh sensors (LDRs) se physical inputs ko read karta hai aur process karne ke baad motor chalane ke liye physical output generate karta hai.

- Light Dependent Resistors (LDRs): * Yeh light sensors hain jo tracking mechanism ke liye zaroori hain.

+1

  - Inhein solar panel ki opposite sides par lagaya jata hai taake roshni ki intensity ko measure kiya ja sakay.

- Relay Module: * Yeh ek electrical switch ki tarah kaam karta hai.

  - Yeh Arduino ke low-power digital signal aur high-current DC motor ke darmiyan ek bridge ka kaam karta hai taake Arduino damage na ho.

+1

- DC Motor: * Yeh motor cleaning action perform karne ke liye wiper ya brush ko solar panel par sweep (chalanay) ke liye use hoti hai.

- Lithium-ion Battery: * Yeh 3.7V ki battery pooray system ko main electrical power provide karti hai.

- DC-DC Boost Converter Module: * Kyun ke Arduino aur relay ko proper kaam karne ke liye exact 5V chahiye hota hai, is liye yeh module battery ke 3.7V ko step-up (boost) karke stable 5V mein convert karta hai.

- Transistor: * Yeh ek semiconductor switch ke tor par kaam karta hai jo Arduino se low-power signal ley kar relay module ko activate (trigger) karta hai.

- Breadboard & Jumper Wires: * Breadboard components ko place karne aur jumper wires un ke darmiyan electrical connections banane ke liye istemaal hotay hain.

+1

- Resistors & Capacitors: * Resistors current ko control karne (Ohm's law ke mutabiq) aur LDRs ke sath circuit banane ke liye use hotay hain. Capacitors voltage fluctuations ko smooth karne ke liye circuit mein lagaye jate hain.

+3

## 3. Working Principle Explained

Is system ke do main working parts hain:

### A) Solar Tracking Mechanism (Light Response)

- Solar tracking feature yeh ensure karta hai ke solar panel hamesha suraj ke samnay rahay taake maximum power generate ho sakay.

- Panel ke opposite sides par do LDRs lagay hotay hain.

- Jab suraj ki roshni (ya testing ke doran flashlight) setup par parti hai, to jis LDR par light intensity zyada hoti hai, us ki electrical resistance drop (kam) ho jati hai.

- Resistance kam honay se ek voltage difference peda hota hai jise control system (Arduino) detect kar leta hai.

- System foran motor ko activate karta hai jo panel ke rukh ko tab tak rotate karti hai jab tak dono LDRs par parnay wali roshni barabar (equal) na ho jaye. Is tarah panel saara din light source ko actively follow karta hai.

+1

### B) Automatic Cleaning System (Arduino Controlled)

- Arduino Nano automatic cleaning system ko control karta hai taake panel par mitti jama na ho.

- Arduino ko is tarah program kiya jata hai ke yeh ek specific interval (timer code) ke baad ya phir panel ki output power mein bohot baray drop ko detect karke cleaning cycle shuru karta hai.

- Jab cleaning ka time hota hai, to Arduino ek low-power digital signal Transistor ko bhejta hai, jo aagey Relay Module ko switch-on kar deta hai.

- Relay circuit ko bridge karti hai, jis se high current direct DC Motor mein jata hai.

- Motor ghoomti hai aur panel par lagay wiper ya brush ko sweep karti hai jis se dhool saaf ho jati hai.

- Jaise hi programmed cleaning time khatam hota hai, Arduino signal stop kar deta hai, relay off ho jati hai aur motor ruk jati hai.

## 4. Relation of Arduino to Applied Physics

Viva mein yeh pucha jata hai ke is project ka Physics se kya taluq hai. Aap yeh points detail mein explain kar sakte hain:

- Solid-State Physics and Semiconductors: Arduino ka jo main microcontroller chip hai, woh silicon se bana hai jo ek semiconductor material hai. Is chip ke andar microscopic transistors hotay hain. Chip ke andar binary data (0s and 1s) ko represent karne ke liye electrons ka jo flow control kiya jata hai, woh direct solid-state physics ka application hai.

+1

- Electricity & Circuits (Ohm’s Law): System mein components ko safe rakhne aur unhein bina burn out kiye chalane ke liye voltage aur current ka control Ohm's Law ke principle par hota hai:

- Light Physics (Photoelectric / Photoconductive Effect): Jab LDRs light ko detect karte hain, to physics ka principle apply hota hai. Light energy (photons) jab sensor surface par parti hai, to woh material ki electrical resistance ko physically change kar deti hai.

- Motion (Electromagnetism): Jab Arduino cleaning ke liye DC motor ko turn on karta hai, to electricity motor ke andar ek magnetic field generate karti hai. Yeh magnetic field physical movement (torque) peda karti hai jis se wiper spin karta hai. Yeh electrical energy ko mechanical energy mein convert karne ka electromagnetic usool hai.

+2